Table of contents

1. Boot to BIOS:	2

Step 1: Reboot LattePanda	2

1.	With Ubuntu	2

2.	With Window	3

Step 2: Enter Bios	5

2. Setting for auto wake-up on power:	6

Solution 1: For BIOS version 2017	6

Solution 2: For others BIOS	6

3. Optional - Setting for auto wake-up on LAN network:	7

Solution 1: For BIOS version 2018	7

Solution 2: For others BIOS	7

4. Optional - Setting for auto wake-up on exactly time:	8

Solution 1: For BIOS version 2018	8

Solution 2: For other BIOS	8

# 1. Boot to BIOS:

#### Step 1: Reboot LattePanda

##### With Ubuntu

To use terminal to reboot LattePanda, enter the following command:

- sudo systemctl reboot now

Press Enter

Or use interface to reboot LattePanda:

Go to the top right corner, click on the gear symbol and select shutdown from the drop-down menu

Choose the left icon

##### With Window

To use command:

Key combination Windows logo key + R to open the Run dialog box

Success screen

Enter the following command to reboot LattePanda after 10s:

- shutdown /r /t 10

Press Enter

Success screen

Or use interface to reboot LattePanda:

Click on the microsoft icon on the bottom left screen and follow 3 steps as shown below:

Do it sequentially from step 1 to 3

Success screen

#### Step 2: Enter Bios

Press ‘DEL’ key continuously to enter into BIOS menu before you see the LattePanda logo on the splash screen

Success screen

# 2. Setting for auto wake-up on power:

Depending on the bios version that LattePanda uses, there are various solutions to this issue. Try each one till you find the suitable solution:

#### Solution 1: For BIOS version 2017

Source: https://m.blog.naver.com/PostView.naver?isHttpsRedirect=true&blogId=pknujjang&logNo=220976659177

Path: BIOS Setup → Boot → Machine Status AC/Battery In

From the part 1 success screen:

- Go into Boot → Machine Status AC/Battery In

- Change it to Power On

Success Screen

#### Solution 2: For others BIOS

Source: http://docs.lattepanda.com/content/3rd_delta_edition/bios/

Path: BIOS Setup -> Advanced -> Power Management -> AcPowerLoss

From the part 1 success screen:

- Go into Advanced → AcPowerLoss

- Change it to Power On

Success Screen

# 3. Optional - Setting for auto wake-up on LAN network:

Depending on the bios version that LattePanda uses, there are various solutions to this issue. Try each one till you find the suitable solution:

#### Solution 1: For BIOS version 2018

Path: BIOS Setup → Advanced → Wake on Lan

From the part 1 success screen:

- Go into Advanced → Wake on Lan

- Change it to Enable

Change from Disable to Enable by press Enter

#### Solution 2: For others BIOS

Path: BIOS Setup → Advanced → Wake on Lan

From the part 1 success screen:

- Go into Advanced → Wake from Lan

- Change it to Enable

Change from Disable to Enable by press Enter

# 4. Optional - Setting for auto wake-up on exactly time:

Depending on the bios version that LattePanda uses, there are various solutions to this issue. Try each one till you find the suitable solution:

#### Solution 1: For BIOS version 2018

Path: BIOS Setup → Chipset → Wake system from S5

From the part 1 success screen:

- Go into Chipset → Wake system from S5

Change it to Enable and set exactly time:

Change from Disable to Enable and set time to auto power on

#### Solution 2: For other BIOS

Path: BIOS Setup → Advanced → Wake on RTC

From the part 1 success screen:

- Go into Advanced → Wake On RTC

Change it to Enable and set exactly time:

Change from Disable to Enable and set time to auto power on